1.0) Introduction

This is a benchmarking program to measure the speed of executing some of

the DIPC system calls in your system (computers and networks).

2.0) How it works

The program consists of two executable files: sysbench and sysmark. The

sysbench program sets up a shared memory, a message queue, and a semaphore

set. sysmark is then started by sysbench to make the system calls. The place

where sysmark should be executed is determined by the contents of the 'sys_s'

script. You could choose sysmark to be executed on the same machine as

sysbench, or you could use the rsh command to run it in another computer. In

this case just make sure both computers are in the same DIPC cluster.

The tested system calls are:

semget()

shmget()

msgget()

semctl() with the IPC_STAT command

semctl() with the IPC_SET command

msgctl() with the IPC_STAT command

msgctl() with the IPC_SET command

shmctl() with the IPC_STAT command

shmctl() with the IPC_SET command

msgsnd() with 1, 10, 100, 500, 1000, 2000 and 4000 byte messages

msgrcv() with 1, 10, 100, 500, 1000, 2000 and 4000 byte messages

semop()

The first three system calls are performed only once. The rest are performed

at least once, and if finished in less than 1 second, they are repeated

until at least 1 second has passed. This method is used to get a more reliable

result on fast computers and/or networks.

The results are then sent to sysbench using a System V messages, and are

displayed to the user.

Note: With the exception of the first three cases, you should wait for at

least 1 second for the results of each test to become available. Expect a

longer time if your network is not fast enough.

3.0) Installing it

To be able to use the program on your system, do the following:

3.1) Compile sysbench and sysmark. Do this by typing make.

3.2) Place one instance of sysmark file in other machines. They should be

connected over a TCP/IP network to the current machine.

3.3) Modify the sys_s script and change it according to your system's

configuration and IP addresses. Make sure that rsh can find the sysmark

program.

4.0) Running it

You may not be able to use rsh when you are root, So you'd better not be the

root. Now just type sysbench. Note that you may have to wait a while for the

program to do its work and finish.