Release notes for gdb-4.16 bdm patches

gm 20.07.96

A. What has changed since 4.13?

===============================

1. GDB default init file name

The gdb initfile has been renamed from .gdbinit to .gdbinit68. This was

to avoid conflicts on my many sessions debugging gdb-bdm with gdb.

2. Access to the supervisor registers

bdm_set and bdm_get are gone. Now, you can query and set the supervisor

registers of the CPU32 with their standard names, i.e.:

$ppc, $usp, $ssp, $sfc, $dfc, $atemp, $far, $vbr

However, if you manipulate the registers, be shure, you know what you are

doing, and see the warnings in the 4.13-release-notes. Some registers are

used by GDB itself.

This feature opens quite some options:

You (probably, I have not yet tried) can skip the bdm-macro facility.

Just use a gdb-macro script. Access to SIM registers can be accomplished

with e.g.

set var * (u_short *) 0xfffa00 = 0x1234

The processor registers are accessed by their names.

set $a4 = 0x12345678

Use

source <gdb-macro-name>

to execute the gdb-macro file.

To add more flexibility, you can create your own gdb commands with the

`define' command.  Any valid gdb command can be used to implement your user

command. In addition, you can use structuring commands, like `if' and

`while'. And you have access to the whole world of Unix commands via the

`sh' command. If I could get you curious, just take a look into the gdb

manual!

3. bdm_status

bdm_status now puts out some nice human readable target status messages.

e.g.

(bdm-gdb) bdm_status

Connected PowerOK Frozen

(bdm-gdb)

4. Automatic reset on download

Some people had problems because of the automatic reset before download.

This was, because they interactively adjusted the SIM configuration, which

was swiped away before download by the autoreset. Now autoreset is user

selectable with

(bdm-gdb) bdm_autoreset off

automatic reset is off

(bdm-gdb)

No argument shows the status of autoreset, on turns it on, off turns it

off.

Please note, if you switch off autoreset, you should issue a reset by hand.

(Use bdm_reset). This is to enable setting of some registers, which are

only writeable once after reset, otherwise you get a bus error.

5. Target monitor - Time to come up

When I initially started work with gdb-bdm, I just had a BCC board with

the 332bug-monitor in the EPROM. I relied on the montitor for setting up the

basic SIM configuration. So after a reset, I let the monitor time to come

up. As gdb-bdm must now serve romless boards as well, the time is now user

selectable.

(bdm-gdb) bdm_timetocomeup 0

(bdm-gdb) bdm_timetocomeup

time-to-come-up is 0

(bdm-gdb)

The time is in usecs. A time of zero keeps the target in BDM mode after a

reset, and does not temporarily start up the monitor.

Default is a timetocomeup of 60000 and autoreset off.

6. RAM-INIT support

You can enable the pre-download of a `ram_init'-File before download of

the application by a compile time switch. Just add -DSUPPORT_RAMINIT to the

CFLAGS. The ram_init-File is a .coff-File, which contains data downloaded

into the SIM registers of the 332. This feature is for the efi332

development environment, provided by John S. Gwynne. He has made provisions

to create the `ram_init'-file out of the crt0 source file.

An alternative to the ram_init-File are Macrofiles. See the release notes

for 4.13 and the examples in the misc-directory.

7. Reset macro file

Now there is a 3rd macro file, downloaded immediately after reset.

It has the same syntax as the macro files before and after download.

I had trouble with newer Tattletale Model 8, when the watchdog stroke

periodically after switching into bdm mode. The only remedy was to

configure at least some of the SIM registers immediately after reset. This

is done by a Macro file in the current directory called `cpu32init'. Right

now I use a Symlink to the .bdmmb file of the current application. Gdb-bdm

complains, if cpu32init is missing, but this can mostly be ignored.

`cpu32init' gets downloaded on every gdb created reset. This is either

autoreset (if enabled) or interactively issued resets with the command

`bdm_reset'.

8. Download of single coff sections.

Now you can download a single coff section with

(bdm-gdb) load section .data

(bdm-gdb)

If you have not downloaded a full .coff file before, gdb-bdm issues an

error.

This is useful, if you have huge applications, which you want to restart.

Usually, the .data section gets clobbered, when executing code, so to

restart, you have to restore just this section. As .bss gets cleared in

crt0, you can avoid download .text, just .data must be downloaded.

This feature can be used for other purposes with non-standard coff-sections

as well.

9. Power and Cable checking

This feature uses provisions in the PD bdm interface on parallel port pin

19 and 21 to work. If the interface board does provide fixed levels (high

on 19 and low on 21) no error condition can be detected, if the interface

permanently puts wrong levels to the pins, this feature can be switched off

with `bdm_checkcable off'. The `target bdm ' command ignores cable and

power faults, as do commands concerning interface and driver parameters,

but any interaction with CPU32 checks faults before and chokes, if checking

is enabled.

10. bdmlib

Now I have separated the architecture and device driver specific stuff into

a `library' file for use in gdb independent applications. Think about a

stand alone downloader without debugging facilities. I have used bdmlib to

build a flash buring package.

11. Error messages

Now the error and status messages are improved and human readable.

No more `error -615'.

12. Error checking

Now there is more precise error checking on the answers from driver. (still

something to do)

13. Start address selection

If you have downloaded a program, you can select the entry point with

(bdm-gdb) bdm_entry <addr>

(bdm-gdb) bdm_entry

current entry point is 0x23456

This can be of some help when it comes to debug code already in ROM. At

least single stepping through the code is possible, although setting

breakpoints cannot work. Remember, GDB itself sets temporary breakpoints to

accomplish some user commands.

14. Shutting interrupts off when single stepping

There have been problems when single stepping (even a `continue' after a

breakpoint uses implicitly single stepping) was done and an interrupt was

pending: instead of stepping to the next instruction, you found yourself

in the ISR. With `continue', you came right back where you wanted to get

off.

Now, interrupts are turned off, when single stepping, and the interrupt

mask is restored, before execution is released on the target. GDB does this

for you, when issueing a `next', `step', or `continue'.

Attention:

There are situations, where the interrupt masking code can be easily

fooled. It is, when you single step over code, which manipulates the

interrupt mask itself. See the comments in the source code. Any suggestion

to improve this cludge is highly welcome!

15. Improved device driver: auto load module support

Now the driver and the driver's Makefile are set up to enable autoloading

via kerneld. See the latest modules-utilitlies (current version is

modules-2.0). If you find that runtime linking fails with something like

`symbol xyz not found', you probably have not configured you kernel for

autoloading. You can either fix this by recompiling the kernel and the

other kernel modules (preferably), or compile bdm.o in the old way, by

clearing the Makefile variable AUTOLOADING.

16. scetchy support for ICD

There are some preliminary provisions for the ICD interface (AKA P&E

cable). Due to time shortage before my vacation, I could not finish

support for this BDM interface. Dont use it right now. If you have trouble

with the PD bdm interface, try the 5 chip interface. The schematic is

included in this package in misc/bd32new.ps.

17. New major device number

The bdm driver has now an officially registered major device number, which

is unique in the whole Linux world. This was to avoid conflicts with other

device drivers. If you want to upgrade, make shure to create the new device

entries in /dev. See driver/MAKEDEV for a suggestion. Note, the ICD support

is a future expansion.

18. Additional ioctls

There are new ioctls for getting the kernel driver version number and cable

checking.

B. Watchouts

============

All watchouts of the release notes for gdb-4.13 are still valid, until

otherwise noted in this document.

There seems to be less trouble with cable length, due to some changes in

the device driver. Thanks to the work of Eric Norum (see credits below).

Detach now closes the bdm target. This closes the bdm device, which

unloads automatically after a certain time (usually 60 seconds). (This is

true, if you use kerneld for autoloading device drivers)

You can re-enable bdm debugging with another target-command.

There is a compilation conflict concerning variable arguments:

#include <varargs.h> is incompatible with

#include <stdarg.h>

Amazingly, both are from the gcc include directory. (I use gcc-2.7.2)

You will get warnings, if you use both #includes, or you might get

warnings, if you use another native compiler version.

In later ELF systems, libtermcap is no longer there. Instead, use -lcurses

as a substitute in .../gdb-4.16/gdb/Makefile.in instead.

C. Credits and related work:

============================

Credits:

--------

Eric Norum <eric@skatter.USask.Ca>

Eric has ported my 4.13 release to NextStep for 68360, which is a little

different than 68332. Actually, he reimplemented the driver, and most of

the bdm backend in gdb, using my source merely as inspiration. He used the

concept of a driver independent `wrapper' library, which is now in this

release as well. He implemented the access to the bdm commands via ioctls

in which structs are handed over. By this he was able to cope with byte

ordering problems. This feature is not included in this release.  Eric also

owns credit of identifying and fixing some kernel timing problems, too.

John S. Gwynne <jsg@coulomb.eng.ohio-state.edu>

John modified gdb-bdm to support romless boards. These mods are included in

the current release. He also added support for the `ram-init' file.

John spent quite some effort to provide a really nice GNU development

environment for the efi332 project, in which gdb-bdm-4.13 has been

included, and gdb-bdm-4.16 will be included.

Related work:

-------------

Peer Stritzinger <stritzi@Physik.TU-Muenchen.DE>

Thomas Doerfler <td@imd.m.isar.de>

Peer wrote bdm patches for gdb-4.13 at about the same time, than I did.

Though we are with the same University, there has been, and there is no

relation between the two versions of gdb-bdm. The difference is, that

Peer's bdm implementation relies on a protocol converter to translate

RS232-commands to BDM commands. By this, the cross-debugger can run on

virtually any workstation with RS232 interface. The external gadget is

produced and sold by IMD, an engineering consultant office here in Munich.

Thomas is the contact person.  It seems that Peer is currently preparing a

gdb-4.16 upgrade as well.

Loris Caren <loris@caren.demon.co.uk>

Loris developed independently a bdm development environment for the ICD bdm

cable. His emphasis has been on interactive handling of things, so he wrote

some tcl scripts around a stack of programs around a minimal parallel port

device driver. With that, he seems to be able to do amazing things, like

programming flash, downloading target routines, etc.

Jeff Dionne <jeff@ryeham.ee.ryerson.ca>

Andrew Dennison <adennison@swin.edu.au>

Based on my gdb-4.13 bdm-backend, they wrote a tool to program AMD flash

memory. See the efi332 www page for current versions:

http://www.cim.swin.edu.au/~aden/web-docs/efi332

I am working on a flash tool, which is not yet ready for release, and which

makes use of bdmlib.c. It is a generic tool capable to program any brand of

flash or eprom, by using a target resident burner program, which runs in

the 2K internal RAM of the 332.

D. Final remarks

================

I hope, the patches are useful. If you have suggestions for further

improvements, or even patches, send them to

magin@lpr.e-technik.tu-muenchen.de

The official ftp site, you can retrieve updates from, is:

ftp://ftp.lpr.e-technik.tu-muenchen.de/pub/bdm

though there will probably be mirrors.

I will be on a _long_ vacation in Aug,Sept & Oct 96', so I cannot help in

those time. (Anybody in Canada (B.C. and Alberta), who can temporarily

offer me internet access to read my email?) However, don't hesitate to

contact me via email, if you have problems (or success).

Plans for future releases:

- make use of ICD cables (by P&E)

- keep up with the FSF gdb releases :-)

gm, 21.07.96