Motorola 68360 Simulator (sim68360)

===================================

This is Jorand Didier's Motorola 68360 (sim68360) simulator.  It doesn't

contain all of the 68360 devices but it does provide a CPU32 based simulator.

Jorand actually uses (used) GCC and GDB to do development work with

BSVC.  For more information see the file bsvc/misc/sim68360/gcc-gdb.ps.

Here's a note about using the samples for sim68360:

Go in the sample/sim68360 directory and play with the tests: IMPORTANT: for

that to work, you need to define 2 environment variables: BSVC20, which

contains the path to the 'bsvc-2.1' directory and 'M68K_LIB' which contains

the path to the vxworks68 libgcc.a directory.  It can look like

'/usr/local/m68k/lib/gcc-lib/vxworks68/2.7.2'. You've got everything to

test gcc and gdb... enjoy.

The following instructions are not currently implemented in sim68360:

CHK

NBCD

SBCD